A:

// Test Case: Autocorrelation of a Rectangular Pulse Sequence

// Define parameters

N = 20; // Number of samples

x_n = [ones(1,5), zeros(1,N-5)]; // Rectangular pulse sequence with width 5

// Compute the autocorrelation

R_xx = correl(x_n, x_n);

// Create a lag vector for plotting

lags = -N+1:N-1;

// Plot the input sequence

clf; // Clear the current figure

subplot(2,1,1);

bar(0:N-1, x_n, 'b');

xlabel('n (Discrete Time Index)');

ylabel('Amplitude');

title('Input Sequence: Rectangular Pulse Sequence');

// Plot the autocorrelation sequence

subplot(2,1,2);

bar(lags, R_xx, 'r');

xlabel('Lag');

ylabel('Autocorrelation');

title('Autocorrelation of the Rectangular Pulse Sequence');

OUTPUT:

B:

// Test Case: Cross-Correlation of Two Discrete-Time Sequences

// Define parameters

N = 20; // Number of samples

x_n = [ones(1,5), zeros(1,N-5)]; // Rectangular pulse sequence with width 5

delay = 3; // Delay of 3 samples

y_n = [zeros(1,delay), x_n(1:$-delay)]; // Delayed version of x[n]

// Compute the cross-correlation

R_xy = correl(x_n, y_n);

// Create a lag vector for plotting

lags = -N+1:N-1;

// Plot the first sequence

clf; // Clear the current figure

subplot(3,1,1);

bar(0:N-1, x_n, 'b');

xlabel('n (Discrete Time Index)');

ylabel('Amplitude');

title('Sequence 1: Rectangular Pulse Sequence');

// Plot the second sequence

subplot(3,1,2);

bar(0:N-1, y_n, 'g');

xlabel('n (Discrete Time Index)');

ylabel('Amplitude');

title('Sequence 2: Delayed Rectangular Pulse Sequence');

19

// Plot the cross-correlation sequence

subplot(3,1,3);

bar(lags, R_xy, 'r');

xlabel('Lag');

ylabel('Cross-Correlation');

title('Cross-Correlation between Sequence 1 and Sequence 2');

OUTPUT: